SOUTH WEST DESIGN AFRICA LIMITED (SWDA)

RENAISSANCE CONTRACT / TENDER NO.: CW-68582 / CW-515913

RISK ASSESSMENT & HAZARD REGISTER

Extracted & Reissued from the SWDA Renaissance 18-Tank Cleaning Project HSE Plan (JHSA)

| FIELD | DETAIL |
| --- | --- |
| Document Title | SWDA Risk Assessment & Hazard Register |
| Document Control No. | SWDA-HSE-HAZREG-001 |
| Revision | 00 |
| Source | Extracted and reissued from Attachment 1 (Job Hazard & Safety Analysis — JHSA) of the Renaissance ATC/NME Tank Cleaning Contract HSE Plan |
| Applicability | SWDA Zero-Man Entry / Auto-Tank Cleaning (ATC) and Sludge Removal operations, Renaissance 18-Tank Cleaning Project |
| Related HSE-MS Reference | Corporate HSE Manual Section 4.0 (Hazard Identification, Risk Assessment & JSA) |
| Custodian | SWDA Regulatory Compliance Manager (Q-HSE Coordinator) |

PREPARED BY: Sonia Somuvie — Regulatory Compliance Manager, SWDA

REVIEWED BY: Engr. Ned Jessup, P.E. — COO / ED & Project Director, SWDA

ENDORSED BY: Chief Executive Officer / Managing Director, South West Design Africa Limited

# 1.0  Purpose & Scope

This Register consolidates the hazards identified in the Job Hazard & Safety Analysis (JHSA) of the Renaissance 18-Tank Cleaning Project HSE Plan into a standalone Risk Assessment & Hazard Register, for ongoing reference, review, and Management oversight independent of the full HSE Plan document. It covers the Zero-Man Entry / Auto-Tank Cleaning (ATC) process and associated sludge removal, transfer, and de-mobilization activities.

# 2.0  Risk Assessment Methodology

Each hazard is assessed for Consequence (C) and Likelihood (L) using a 1–5 scale, producing a Risk Rating (RR) via the Risk Assessment Matrix (RAM):

| CONSEQUENCE (C) | DESCRIPTION |
| --- | --- |
| 5 | Fatality, Public Hospitalization, or Severe Health Effects |
| 4 | Permanent Disability, Multiple Hospitalizations, or Major Health Effects |
| 3 | One or More Lost Time Workday Cases or Significant Health Effects |
| 2 | Medical Treatment with Restricted Duty or Medium Health Effects |
| 1 | Medical Treatment, Minor Health Effects, First Aid Case, or Less |
| LIKELIHOOD (L) | DESCRIPTION |
| 4 | Frequent |
| 3 | Probable |
| 2 | Rare |
| 1 | Remote |
| — | Improbable (lowest) |
| RISK RATING (RR) | CATEGORY | REQUIRED ACTION |
| I (1–4) | Low | No mitigation required |
| II (5–10) | Medium | Controls verified — ALARP |
| III (12–16) | Significant | Mitigate with priority |
| IV (17–25) | High | Mitigate with highest priority; do not proceed if still Category III/IV after mitigation |

Per the HSE Plan's risk methodology: achieve a Category I or II before proceeding with any task; do not proceed if the risk category remains III or IV after mitigation.

# 3.0  Hazard Register

The table below lists all 35 hazards identified in the source JHSA, with Initial and Residual Consequence (C), Likelihood (L), and Risk Rating (RR), and a summary of key controls/mitigation. Full control detail for each hazard remains in the project HSE Plan JHSA (Attachment 1).

| # | HAZARD | C | L | RR | KEY CONTROLS / MITIGATION | RES. C | RES. L | RES. RR |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Chemicals — Maintenance Oils, Cleaning Chemicals | 3 | 2 | II | SDS available; UN-approved packaging; PTW; spill pallet/kit; PPE | 3 | 1 | I |
| 2 | Tank Cleaning Diluent — Crude Oil/Gas Oil, Detergent | 5 | 4 | IV | ATC employed; bunded site; inert (N2) atmosphere; ATEX equip; O2 auto-shutdown >8% | 5 | 2 | II |
| 3 | Hot Surfaces, Hot Water & Steam | 3 | 4 | III | PTW; flame-retardant coveralls; heat-resistant gloves; pipe covers | 2 | 2 | I |
| 4 | Fire on Site | 3 | 4 | III | PTW; fire extinguishers placed on site | 3 | 2 | I |
| 5 | H2S (Hydrogen Sulfide) | 4 | 3 | III | Fixed H2S monitors/alarms; personal monitors; BA use; PTW; JSA | 3 | 3 | II |
| 6 | Live Services — Electricity, Steam, Nitrogen, Air | 4 | 3 | III | PTW; LOTO; line-breaking procedure; O2 monitors; works cordon | 3 | 2 | II |
| 7 | Electrical 220V/400V 50Hz System | 4 | 3 | III | PTW; ATEX-rated/armoured cables; inspections; breaker protection | 3 | 2 | II |
| 8 | Confined Space Entry (tank roof, nitrogen purging) | 5 | 3 | III | CSE PTW; trained CSE operatives; SCBA/escape sets; 4-gas detectors; no lone working | 5 | 2 | II |
| 9 | Charging Tank with Nitrogen | 5 | 3 | III | PTW; O2 monitoring/alarm; whip-check wires; stop N2 fill when roof entry required | 3 | 3 | II |
| 10 | Automatic Tank Cleaning Process — BLABO NME | 5 | 3 | III | PTW; O2 monitoring; ATEX equipment; auto shutdown >8% O2; pressure relief valves | 3 | 3 | II |
| 11 | Heater Operation | 5 | 3 | III | PTW; auto back-flush; safety relief valve; temp-sensor alarm/shutdown | 3 | 3 | II |
| 12 | Distributors on Roof | 5 | 3 | III | PTW; O2 monitoring; fail-safe valve positioning; SNS auto-stop | 3 | 3 | II |
| 13 | Booster Pumps | 5 | 3 | III | PTW; pressure alarm; non-return valve; double mechanical seal | 3 | 3 | II |
| 14 | Filter Cleaning | 5 | 3 | III | PTW; appropriate PPE; two-person lifting; no ignition sources | 3 | 3 | II |
| 15 | Sampling at Sample Points | 5 | 3 | III | PTW; appropriate PPE/containers; good housekeeping; no ignition sources | 3 | 3 | II |
| 16 | Removal of BLABO Nozzles on Roof | 5 | 3 | III | CSE PTW; full SCBA; isolate BLABO system prior; O2 <8% before proceeding | 3 | 3 | II |
| 17 | Laboratory Testing | 5 | 3 | III | PTW; appropriate PPE; good housekeeping; no ignition sources | 3 | 3 | II |
| 18 | Tri-Decanter Operations | 5 | 3 | III | PTW; flame-retardant coveralls; hearing protection; vibration/overload auto-shutdown | 3 | 3 | II |
| 19 | Recovered Oil Tank Operations | 3 | 2 | II | PTW; remote bund; level sensors; auto pump slow/stop at set-point | 2 | 2 | I |
| 20 | Replacing Solids Container | 4 | 2 | II | PTW; spill pallet; fill monitored to ~75%; forklift movement of filled IBC | 2 | 2 | I |
| 21 | Forklift Operations | 5 | 2 | II | Certified forklift; trained drivers only; no fork riding; flashing beacons | 2 | 2 | I |
| 22 | Refuelling of Plant & Equipment — Diesel | 5 | 2 | II | PTW; designated PPE; toolbox talk; spill tray/kit at point of filling | 3 | 2 | II |
| 23 | Working at Heights | 5 | 2 | II | Walkways/platforms/safety gates; scaffold per plan; tool tethers; load inspection | 3 | 2 | II |
| 24 | Moving Plant & Equipment | 5 | 2 | II | Site roads/signage; speed limits; banksman; reverse-in/drive-out policy | 3 | 2 | II |
| 25 | Access to Tank Roof | 3 | 2 | II | Classified confined space when N2 filling; CSE attendant; escape set/4-gas | 3 | 2 | II |
| 26 | Asbestos — If Suspected or Found | 3 | 3 | II | Asbestos register/management plan; PTW; safe removal techniques; notify RAEC Safety | 3 | 2 | II |
| 27 | Line Breaking — Opening Pipelines/Flanges | 3 | 4 | III | PTW; LOTO; drain lines; zero pressure check; spill tray; visor/goggles | 3 | 2 | II |
| 28 | Removal of Solids from Tank (post water-wash entry) | 5 | 3 | III | CSE permit; LEL <10% (work at ~5%); gas detection; jetting w/ auto-shutoff; rotate entrants | 5 | 2 | II |
| 29 | Transfer of Waste — ADR Unit to IBCs/Drums | 5 | 3 | III | ADR vehicles; earthing; gas detectors; bunded transfer area; no lone working | 5 | 2 | II |
| 30 | De-Mobilisation of ATC Equipment (pipes/hoses/cables) | 5 | 3 | III | PTW; LOTO; spill trays at joints; two-person lift for heavy items | 3 | 3 | II |
| 31 | Emptying Skimming & Decanter Tanks | 5 | 3 | III | PTW; LOTO; remote bund; spill trays when opening joints | 3 | 3 | II |
| 32 | Removal of Riser Pipe | 5 | 3 | III | PTW; radio watchman; certified crane/banksman; harness at wind girder; weather monitoring | 3 | 3 | II |
| 33 | Lifting of Equipment from Roof | 5 | 3 | III | PTW; CSE attendant; certified crane/banksman; harness; weather-dependent stop-work | 3 | 3 | II |
| 34 | Scaffold Access | 4 | 3 | III | RAEC-certified scaffold; weekly inspections; scaff-tag check before use | 3 | 2 | II |
| 35 | Loading of Containers/Equipment onto Transport Units | 5 | 3 | III | Valid permit; toolbox talk incl. crane/truck drivers; banksman; CBR-tested ground; lifting plan | 3 | 3 | II |

# 4.0  Application & Review

- This Register is reviewed annually, or immediately following any incident, near-miss, or change in scope, process, personnel, or equipment (per Corporate HSE Manual Management of Change provisions).

- Any hazard whose Residual Risk Rating remains III or IV after mitigation is escalated to the Regulatory Compliance Manager and Project Director for further control before work proceeds.

- This Register is a living document — new hazards identified during operations are added following the same C/L/RR methodology in Section 2.0.

# 5.0  Managing Director's Endorsement

This Risk Assessment & Hazard Register, extracted and reissued from the Renaissance 18-Tank Cleaning Project HSE Plan, is endorsed as the Company's current consolidated hazard record for these operations.

SIGNED: ______________________________          DATE: ______________

Chief Executive Officer / Managing Director

South West Design Africa Limited (SWDA)