SWDA RISK EVALUATION & MANAGEMENT REGISTER

SOUTH WEST DESIGN AFRICA LTD. (SWDA) SWDA – Renaissance Africa Energy Company (RAEC) Tank Cleaning Project

| Project | Provision of Zero-Man Entry & Auto-Tank Cleaning (ATC/NME), Sludge Treatment, Oil Recovery & Waste Management Services at Selected Renaissance Facilities |
| --- | --- |
| Contract/Tender No. | Renaissance-CW-68582 / CW-515913 |
| Sites Covered | Century, Agbami & Utorogu |
| Date Compiled | [Insert Date] |
| Ref. No. | SWDA-RISK-REG-[XX] |

RISK RATING KEY: L = Likelihood (1–5) | S = Severity (1–5) | RR = Risk Rating (L×S) | 1–4 Low | 5–9 Medium | 10–15 High | 16–25 Extreme

| S/N | Site | Risk/Hazard Identified | Potential Consequence | L | S | RR | Risk Control / Mitigation Measure | Responsible | Residual RR | Status | |---|---|---|---|---|---|---|---|---|---| | 1 | Century | Manual tank entry bypassing NME/ATC system | Confined-space fatality/injury | 2 | 5 | 10 | Enforce No-Man-Entry protocol; PTW & confined-space entry permit required for any exception; supervisor sign-off | Lead Entrant Supervisor | 2 | Open | | 2 | Century | Hydrocarbon vapour exposure during suction/skimming operations | Toxic exposure, fire/explosion | 3 | 5 | 15 | Continuous gas monitoring; breathing apparatus provided; hot-work permit control; ignition source management | Site HSE Officer | 5 | Open | | 3 | Century | Poor housekeeping at waste-skip staging area | Slips/trips, environmental contamination | 3 | 2 | 6 | Scheduled housekeeping inspections; designated waste segregation points | Site HSE Officer | 2 | Closed | | 4 | Agbami | Workforce grievance/industrial unrest during Mgmt. & Workers' Forum | Work stoppage, reputational risk | 2 | 3 | 6 | Regular engagement forums; grievance-reporting channel; transparent communication with workforce | Senior Project Manager | 2 | Open | | 5 | Agbami | Equipment failure – ATC modules (M1–M4) | Operational delay, uncontrolled release | 2 | 4 | 8 | Preventive maintenance schedule; spare parts availability; daily equipment inspection checklist | Lead Entrant Supervisor | 4 | Open | | 6 | Agbami | Logistics/road transport to site | Road traffic accident (RTA) | 3 | 4 | 12 | Journey Management Plan; certified drivers; vehicle inspection; speed limit enforcement | Site HSE Officer | 4 | Open | | 7 | Utorogu | Community unrest / disruption of works | Project delay, safety/security risk to personnel | 2 | 4 | 8 | Regular Community Engagement Meetings; Community Liaison Officers; local workforce inclusion per Personnel Plan | Community Relations Officer | 3 | Open | | 8 | Utorogu | Local content/workforce selection disputes | Reputational risk, community grievance | 2 | 3 | 6 | Transparent, CDC-agreed selection process; grievance-redress log | Community Relations Officer | 2 | Open | | 9 | Utorogu | Environmental impact – spill/waste near community | Environmental contamination, community harm | 2 | 5 | 10 | Spill-prevention controls; Waste Management Plan; emergency spill-response kit on site | Site HSE Officer | 3 | Open |

MANAGEMENT REVIEW

This Register shall be reviewed monthly by the SWDA HSE Review Committee, and updated whenever new hazards are identified, control measures change, or following any incident/near-miss on the above sites. Residual risks rated "High" or above shall be escalated immediately to the MD and Senior Project Manager for further action.

ENDORSEMENT

This SWDA Risk Evaluation & Management Register has been reviewed and is hereby endorsed as the Company's current risk record for the Century, Agbami and Utorogu work sites.

SIGNED: ______________________________ Mr. Gliffeth Wonuigwe Group CEO / Managing Director – South West Design Africa Ltd. (SWDA) Date: ______________________________