SOUTH WEST DESIGN AFRICA LIMITED (SWDA)

PTW, LOTO & EQUIPMENT INTEGRITY,

CALIBRATION & CERTIFICATION GENERAL PROCEDURES

| FIELD | DETAIL |
| --- | --- |
| Document Title | SWDA PTW, LOTO & Equipment Integrity, Calibration & Certification General Procedures |
| Document Control No. | SWDA-HSE-PTWLOTO-001 |
| Applicability | All SWDA offices, projects, contracts and sites; all employees and sub-contractors |
| Related HSE-MS Reference | Corporate HSE Manual Section 5.0 (Permit to Work System); Section 8.0 (Training of Critical Roles & Management) |
| Custodian | SWDA Regulatory Compliance Manager (Q-HSE Coordinator); Equipment Maintenance Manager (equipment integrity) |

PREPARED BY: Sonia Somuvie — Regulatory Compliance Manager, SWDA

REVIEWED BY: Ernest Okorie — Equipment Maintenance Manager, SWDA

ENDORSED BY: Chief Executive Officer / Managing Director, South West Design Africa Limited

# 1.0  Purpose & Scope

This document sets out SWDA's general procedures for Permit to Work (PTW), Lockout/Tagout (LOTO), and Equipment Integrity, Calibration & Certification, across all offices, projects, contracts and sites. It expands on Corporate HSE Manual Section 5.0 (Permit to Work System) with full operating detail for these three closely linked control disciplines.

# 2.0  Permit to Work (PTW) System

## 2.1  PTW Principles

No high-risk activity — confined-space/tank entry, hot work, excavation, lifting, or electrical isolation — commences without a valid, site-specific Permit to Work, authorized by the Client (where applicable) and countersigned by the Site Safety Manager.

## 2.2  Permit Types

| PERMIT TYPE | APPLIES TO |
| --- | --- |
| Confined Space Entry (CSE) Permit | Tank/confined-space entry, including Zero-Man Entry/ATC-supported entry |
| Hot Work Permit | Welding, cutting, grinding, or any spark/flame-producing activity |
| Electrical Isolation Permit | Work on or near live electrical systems, requiring LOTO |
| Lifting Permit | Crane lifts, rigging, and mechanical lifting operations |
| Excavation Permit | Ground-breaking or excavation work |
| Line-Breaking Permit | Opening pipelines, flanges, or pressurized systems |

## 2.3  Authorization, Issuance & Closure

- Permits specify: validity period, authorized personnel, required gas-test results (where applicable), isolation/LOTO requirements, and standby rescue arrangements.

- Permits are displayed at the worksite for the duration of the activity and are void if conditions change (weather, personnel, equipment) until re-assessed and re-authorized.

- On completion, the permit is formally closed by the issuing authority, confirming the area is left safe and any isolations have been correctly restored.

# 3.0  Lockout/Tagout (LOTO) Procedure

LOTO ensures hazardous energy sources (electrical, mechanical, hydraulic, pneumatic, nitrogen/inert gas) are isolated and cannot be re-energized while work is in progress.

## 3.1  LOTO Steps

| STEP | ACTION |
| --- | --- |
| 1 | Identify all energy sources affecting the equipment/system to be worked on |
| 2 | Notify affected personnel that isolation is about to occur |
| 3 | Shut down the equipment/system using normal operating controls |
| 4 | Isolate all identified energy sources (electrical breakers, valves, mechanical stops) |
| 5 | Apply a personal lock and tag to each isolation point — one lock per person working on the equipment |
| 6 | Verify zero energy state (test, bleed residual pressure, confirm de-energized) before work begins |
| 7 | Perform the work |
| 8 | On completion, each individual removes their own lock/tag only; never remove another person's lock |
| 9 | Confirm the area is clear and safe before restoring energy and returning equipment to service |
| GROUP / MULTIPLE LOCKOUT RULE Where more than one person or crew is working on the same isolated system, a group lockout box/hasp is used so that each individual applies their own personal lock. Energy may not be restored until every individual lock has been removed by its owner — no exceptions, even under schedule pressure. |

## 3.2  Roles

- Authorized Isolator: performs the physical isolation and verifies zero energy state.

- Permit Holder / Site Safety Manager: confirms LOTO is correctly applied before authorizing work to begin under the associated PTW.

- Equipment Maintenance Manager: maintains the LOTO equipment (locks, tags, hasps) and audits LOTO compliance during equipment maintenance activities.

# 4.0  Equipment Integrity, Calibration & Certification

## 4.1  Equipment Covered

| EQUIPMENT CATEGORY | EXAMPLES |
| --- | --- |
| Gas detection equipment | Portable and fixed gas monitors (O2, flammable gas, H2S) |
| Confined-space & rescue equipment | SCBA/SAR sets, harnesses, retrieval/winch equipment, tripods |
| Lifting equipment | Cranes, slings, shackles, hoists, forklifts |
| Tank cleaning systems | BLABO/ATC equipment, booster pumps, heaters, tri-decanters, filters |
| Electrical equipment | ATEX-rated tools, armoured cables, breakers |
| PPE with certification requirements | Fall-arrest harnesses, respiratory protection |

## 4.2  Calibration & Certification Schedule

| EQUIPMENT | FREQUENCY | RESPONSIBLE |
| --- | --- | --- |
| Gas detectors (portable & fixed) | Calibration per manufacturer schedule, minimum quarterly; bump-tested before each use | On-Site HSE Officer / Equipment Maintenance Manager |
| Lifting equipment (cranes, slings, rigging) | Certified inspection per statutory/Client requirement, minimum annually | Equipment Maintenance Manager; certified third-party inspector |
| SCBA/SAR & rescue equipment | Inspected before every use; full service per manufacturer schedule | Rescue Access Team; Equipment Maintenance Manager |
| Tank cleaning systems (BLABO/ATC, pumps, heaters) | Scheduled maintenance and pressure-relief valve testing per OEM schedule | Equipment Maintenance Manager; Tank Clean Systems Operators |
| Fall-arrest harnesses | Visual inspection before each use; certified inspection annually | All users; Equipment Maintenance Manager |

## 4.3  Inspection & Tagging

- All equipment subject to calibration/certification carries a visible, current inspection tag showing last inspection date and next due date.

- Equipment with an expired or missing inspection tag is removed from service immediately and may not be used until re-certified.

- Any equipment found defective during use is tagged out, reported to the Equipment Maintenance Manager, and replaced or repaired before further use.

## 4.4  Records

- Calibration and certification records are maintained by the Equipment Maintenance Manager and made available for Client or regulatory audit on request.

- Equipment integrity status is a standing item at the HSE-MS Quarterly Review Meeting, alongside PTW and LOTO compliance audit results.

# 5.0  Roles & Responsibilities

- On-Site HSE Officer: verifies PTW compliance and gas-detector calibration status before authorizing high-risk work.

- Equipment Maintenance Manager: owns equipment integrity, calibration and certification records; maintains LOTO equipment; audits compliance.

- Lead Field / Entrant Supervisors: enforce PTW and LOTO compliance for their crews; verify equipment tags before use.

- All Employees & Sub-contractors: apply personal locks/tags correctly; never bypass PTW, LOTO, or equipment tag-out controls.

# 6.0  Managing Director's Endorsement

These PTW, LOTO & Equipment Integrity, Calibration & Certification General Procedures are endorsed as the Company's binding standard across all SWDA operations.

SIGNED: ______________________________          DATE: ______________

Chief Executive Officer / Managing Director

South West Design Africa Limited (SWDA)